# A Saturation-Aware Dual-Stage Framework for Intensity-Driven Adaptive PTW: Prioritizing Operational Safety and Site-Response Robustness in the Java-Sumatra Region

## Authors

Hanif Andi Nugraha¹\, Dede Djuhana¹, Adhi Harmoko Saputro¹, Sigit Pramono*²

¹ Department of Physics, Faculty of Mathematics and Natural Sciences, Universitas Indonesia, Depok, Jawa Barat, Indonesia

² The Agency for Meteorology, Climatology and Geophysics of the Republic of Indonesia (BMKG), Jakarta, Indonesia

\*Correspondence: hanif.andi@ui.ac.id

## Abstract

The dataset contains 34,033 three-component accelerometric traces from 329 earthquakes (M 4.0–7.3) recorded at 388 stations across the Sunda subduction zone, specifically focusing on the Java-Sumatra study area (95°–115°E, 11°S–6°N). We present a Saturation-Aware Dual-Stage framework that adaptively selects the PTW through a novel Feature Dichotomy paradigm — systematically exploiting the distinct physical behaviors of saturating and non-saturating P-wave intensity measures (IMs). Stage 1 employs an XGBoost "Intensity Gate" classifier using predominantly the saturating parameter τ_c (predominant period, 51% feature importance) to route seismic traces into three operational intensity classes (Weak, Felt, and Damaging), while Stage 2 leverages the non-saturating Cumulative Velocity Absolute Displacement (CVAD, 89.5% importance) to predict spectral acceleration Sa(T) across 103 periods (T = 0.1–5.0 s). An on-site epicentral distance estimator trained on 30 P-wave features (Zhang et al., 2024) feeds an Intensity × Distance lookup table constrained by the S-wave arrival golden time, ensuring that the selected PTW never compromises warning feasibility. Using event-grouped 5-fold cross-validation on the study area dataset, the pipeline achieves a composite R² = 0.7309 and RMSE = 0.5181 log₁₀ m/s² for fully operational deployment — where both intensity class and epicentral distance are predicted solely from the initial 3-second P-wave signal at a single station. This result demonstrates that the adaptive framework maintains engineering-grade accuracy while prioritizing operational safety through a 55× improvement in damaging recall compared to standard source-aware models. The Feature Dichotomy emerges naturally from P-wave physics and offers a new design paradigm for next-generation EEWS.

Keywords: earthquake early warning, spectral acceleration, adaptive time window, XGBoost, feature dichotomy, Sunda subduction zone, single-station on-site, golden time constraint

## 1. Introduction

### 1.1 Earthquake Early Warning Systems

The Indonesian archipelago sits atop one of the most seismically active regions on Earth, where the Indo-Australian plate subducts beneath the Eurasian plate along the 5,500-km Sunda megathrust at rates of 50–70 mm/yr (Simons et al., 2007; Natawidjaja et al., 2006). This tectonic setting has produced devastating earthquakes including the 2004 M 9.1 Sumatra-Andaman, 2006 M 6.3 Yogyakarta, and 2018 M 7.5 Palu events. Earthquake Early Warning Systems (EEWS) exploit the travel-time difference between fast P-waves (V_P ≈ 6.5 km/s) and slower, more destructive S-waves (V_S ≈ 3.5 km/s) to issue alerts seconds to tens of seconds before strong shaking arrives (Allen and Melgar, 2019; Cremen and Galasso, 2020).

Indonesia's national EEWS, InaTEWS, operated by the Badan Meteorologi, Klimatologi, dan Geofisika (BMKG), currently utilizes a network-based approach requiring multiple station triggers before issuing warnings (Kopp et al., 2008; Münchmeyer et al., 2021). On-site single-station methods offer a complementary approach that can provide faster initial estimates, particularly for near-field events where network triangulation introduces critical delays (Nakamura, 1988; Wu and Kanamori, 2005a). In recent years, regional EEWS implementations have proliferated globally, reflecting distinct tectonic environments and technological constraints. This includes comprehensive nationwide upgrades in Japan (Kodera et al., 2021), offshore observation networks (Dhakal and Kunugi, 2021), and advancing operational systems in Israel (Nof et al., 2021), Switzerland (Massin et al., 2021), Italy (Ladina et al., 2021), and Greece (Bracale et al., 2021). Parallel development is occurring in active zones such as Costa Rica (Porras et al., 2021), southern Iberia (Carranza et al., 2021), British Columbia (Schlesinger et al., 2021), and Sichuan, China (Peng et al., 2021). Comparative studies evaluating these regional algorithms confirm the inherent difficulties of balancing speed and accuracy across diverse geographical configurations (Zuccolo et al., 2021).

In areas with sparse station coverage or lower seismicity, establishing robust dense networks requires substantial capital investment, making optimized single-station algorithms or crowdsourced telemetry networks highly valuable alternative strategies (Parolai et al., 2017; Finazzi, 2020; Velazquez et al., 2020; Ahn et al., 2023a). However, single-station approaches have historically been limited by their lower accuracy compared to network solutions.

### 1.2 From PGA to Spectral Acceleration

Traditional EEWS predict scalar ground motion parameters such as Peak Ground Acceleration (PGA) or Peak Ground Velocity (PGV) (Wu and Kanamori, 2005b; Zollo et al., 2010). While PGA correlates with perceived shaking intensity, it poorly represents the frequency-dependent damage potential of earthquakes on structures (Bommer and Alarcon, 2006). Modern performance-based earthquake engineering requires spectral acceleration Sa(T) at the natural periods of structures: low-rise buildings respond to short-period Sa (T ≈ 0.1–0.5 s), while high-rise buildings and bridges are governed by long-period Sa (T ≈ 1–5 s) (Abrahamson et al., 2014).

The expansion of machine learning in EEWS has been explosive; for instance, modern deep convolution architectures have dramatically improved magnitude estimation (Zhu et al., 2021), and Long Short-Term Memory (LSTM) networks have been successfully applied strictly to early PGA prediction (Hsu and Pratomo, 2022). Progress has even allowed direct mapping of initial warning features to structural drift predictions using machine learning regressors (Iaccarino et al., 2021).

Lara et al. (2023) demonstrated that machine learning can predict Sa(T) at approximately 20 spectral periods from P-wave features, achieving R² = 0.88–0.92 using multi-station network data. However, their approach requires multiple station observations and does not address the PTW optimization problem. Extending EEWS to predict Sa(T) across a comprehensive frequency range (100+ periods) from a single station remains an open challenge.

### 1.3 The P-Wave Time Window Trade-off

The P-wave time window (PTW) — the duration of P-wave signal analyzed before issuing a prediction — presents a fundamental accuracy-versus-timeliness trade-off. Longer windows accumulate more seismic information, particularly low-frequency content that correlates with earthquake magnitude, yielding more accurate predictions (Lancieri and Zollo, 2008; Colombelli et al., 2015). Shorter windows provide earlier warnings but sacrifice prediction quality. Existing EEWS overwhelmingly employ fixed PTW values (typically 3–4 s), applying the same observation duration regardless of earthquake magnitude, distance, or available lead time (Böse et al., 2009; Festa et al., 2008).

This one-size-fits-all approach is fundamentally suboptimal. Small nearby earthquakes generate short-duration P-wave pulses whose diagnostic information is fully captured within 2–3 s, while large distant earthquakes produce extended P-wave trains whose low-frequency characteristics emerge only after 6–10 s (Yamada and Mori, 2009). An adaptive PTW that accounts for event characteristics could simultaneously improve accuracy for large events and preserve timeliness for small ones. The fundamental applicability of initial P-wave data for accurate ground motion estimation has been robustly validated over large datasets (Tsuno, 2021; Wang and Zhao, 2021), while advanced algorithms have begun transitioning toward fully continuous wavefield-based real-time predictions (Hoshiba, 2021; Li et al., 2021).

### 1.4 The Feature Saturation Problem

A critical yet underexplored limitation of P-wave-based EEWS is the saturation behavior of intensity measures (IMs). The predominant period τ_c (Kanamori, 2005) and peak displacement P_d exhibit ceiling effects for large earthquakes (M > 6.0–6.5), where their values plateau rather than continue scaling with magnitude (Brown et al., 2009; Lancieri and Zollo, 2008). This saturation arises from the finite rupture duration: for large earthquakes, the initial P-wave pulse captures only the onset of rupture, and τ_c converges to a maximum value regardless of total magnitude.

Conversely, cumulative parameters such as the Cumulative Absolute Velocity (CAV; Reed and Kassawara, 1990) and Cumulative Velocity Absolute Displacement (CVAD) continue to grow with both magnitude and observation time without theoretical saturation. CVAD, defined as the time integral of the absolute velocity–displacement product, monotonically increases as the P-wave window expands, making it sensitive to both source size and wave propagation effects.

We hypothesize that this saturation dichotomy can be systematically exploited: saturating parameters whose ceiling creates natural category boundaries are well-suited for discrete classification, while non-saturating parameters that maintain continuous scaling are ideal for regression tasks. This paradigm has not been explored in prior EEWS literature.

### 1.5 Distance-Aware Window Selection

The optimal PTW additionally depends on the source-to-site distance through the golden time constraint — the S−P travel-time difference that defines the maximum allowable observation window before shaking onset. Near-field stations (D < 50 km) have golden times as short as 5 s, strongly constraining PTW choices. Far-field stations (D > 300 km) may have golden times exceeding 60 s, providing ample budget for extended observation (Zhang et al., 2024). Recent advances in on-site epicentral distance estimation from P-wave features (Zhang et al., 2024) enable real-time distance-aware PTW selection without requiring network-based source location.

### 1.6 Study Objectives

This study introduces a Saturation-Aware Dual-Stage Adaptive PTW framework with three principal contributions:

Feature Dichotomy Paradigm. The first systematic exploitation of P-wave IM saturation properties in EEWS design: saturating parameters (τ_c) for intensity classification routing, and non-saturating parameters (CVAD) for Sa(T) regression.

Distance-Aware Adaptive PTW. An Intensity × Distance lookup table with golden time constraint that dynamically selects the optimal PTW per trace, maximizing information accumulation while guaranteeing warning feasibility.

Comprehensive Single-Station Spectral Prediction. End-to-end prediction of Sa(T) at 103 spectral periods (T = 0.1–5.0 s) from a single station using only the initial 3-second P-wave signal, validated with rigorous event-grouped cross-validation on 34,033 traces from the Sunda subduction zone.

## 2. Data and Methods

### 2.1 Study Area and Dataset

The Sunda subduction zone extends over 5,500 km from Myanmar through Sumatra, Java, and the Lesser Sunda Islands, accommodating convergence of 50–70 mm/yr between the Indo-Australian and Eurasian plates (Simons et al., 2007). Our dataset comprises three-component (HN*) accelerometric waveforms recorded across this region, compiled into the SeisBench-compatible (Woollam et al., 2022) Sunda v2 (Geomean) dataset.

## Abstract

The dataset contains 34,033 three-component accelerometric traces from 329 earthquakes (M 4.0–7.3) recorded at 388 stations of the BMKG accelerograph network (IA network) within the defined study area (95°–115°E, 11°S–6°N). All waveform data were obtained directly from BMKG's operational recording infrastructure, reflecting real ground motions observed throughout the data acquisition period. Epicentral distances range from 0 to 1,205 km with a median of 384 km, reflecting the sparse station distribution across the Indonesian archipelago and the predominantly offshore location of subduction zone seismicity. The large median distance implies substantial golden times (median ≈ 51 s), providing ample budget for adaptive PTW exploration. The study area filter ensures geographic consistency by excluding 281 stations with missing coordinates, 80 stations east of 115°E (Bali, NTT, Sulawesi), and 22 events with erroneous catalog coordinates.

Table 1. Dataset summary and intensity class distribution.

| Parameter | Value |
| --- | --- |
| Total traces | 34,033 |
| Unique earthquakes | 329 |
| Unique stations (IA-BMKG) | 388 |
| Study area | 95°–115°E, 11°S–6°N |
| Data source | BMKG operational accelerograph network |
| Magnitude range | M 4.0–7.3 |
| Distance range | 0–1,205 km (median 384 km) |
| *Weak* class (PGA < 3.0 Gal) | 32,001 traces (94.0%) |
| *Felt* (3.0 ≤ PGA < 62.0 Gal) | 1,931 traces (5.7%) |
| *Damaging* (PGA ≥ 62.0 Gal) | 101 traces (0.3%) |
| PTW windows | 2, 3, 4, 6, 8 s |
| Sa(T) target periods | 103 (T = 0.1–5.0 s) |

The severe imbalance in the Damaging class (only 101 traces, 0.3%) reflects the inherent rarity of intense ground motions, yet these are precisely the events most critical for EEWS. This imbalance is further amplified by the study area restriction: 74.5% of all recorded traces originate from far-field paths (>200 km), where seismic waves have attenuated substantially before reaching the recording stations. The median PGA of 0.13 Gal (well below the human perceptibility threshold of ~3 Gal) confirms that the dataset is dominated by weak shaking — a faithful representation of the operational reality faced by BMKG's accelerograph network in the Java-Sumatra region.

The spatial distribution of the 329 earthquakes and 388 accelerograph stations within the study area is shown in Figure 1. Event depths are classified following BMKG's standard convention: shallow (≤60 km, red), intermediate (60–300 km, yellow), and deep (>300 km, green). The basemap is rendered using the Rupabumi Indonesia tile service provided by Badan Informasi Geospasial (BIG). This coverage ensures that the pipeline is exposed to a wide variety of path effects and site conditions characteristic of the Indonesian Sunda arc, while maintaining geographic consistency by restricting all data to the Java-Sumatra study region.

[INSERT FIGURE: fig01.png]

Figure 1. Seismicity map of the Java-Sumatra study area overlaid on the BIG Rupabumi Indonesia basemap. Colored circles represent 329 earthquakes (M 4.0–7.3) scaled by magnitude and colored by depth following BMKG convention: shallow ≤60 km (red), intermediate 60–300 km (yellow), deep >300 km (green). Black triangles denote 388 IA-BMKG accelerograph stations. The red rectangle delineates the study area boundary (95°–115°E, 11°S–6°N). The dashed red line indicates the approximate trace of the Sunda Trench. Data period: 2020–2025.

Figure 2 illustrates the dataset's numerical characteristics, including the magnitude–intensity distribution, epicentral distance distribution, and their joint scatter.

[INSERT FIGURE: fig02.png]

Figure 2. Dataset overview (Java-Sumatra Filtered). (a) Magnitude distribution across filtered classes. (b) Hypocentral distance distribution with median focus on the regional study area.

### 2.2 Feature Extraction

For each trace at each PTW, we extract 42 features comprising two complementary sets:

12 Intensity Measures (IMs). These P-wave parameters span the saturation spectrum (Table 2). The predominant period τ_c is computed as:

\tau_c = \frac{2\pi}{\sqrt{r}} \quad \text{where} \quad r = \frac{\int_0^{\text{PTW}} \dot{u}^2(t) \, dt}{\int_0^{\text{PTW}} u^2(t) \, dt}

where u(t) is the vertical displacement and u̇(t) is its time derivative (Kanamori, 2005). τ_c saturates for M > 6.5 due to finite rupture effects but provides excellent discrimination among smaller magnitude categories.

The Cumulative Velocity Absolute Displacement (CVAD) is defined as:

\text{CVAD} = \int_0^{\text{PTW}} |v(t)| \cdot |d(t)| \, dt

where v(t) and d(t) are velocity and displacement waveforms, respectively. CVAD is monotonically increasing with PTW and does not exhibit theoretical saturation, making it ideal for continuous regression.

Table 2. Twelve intensity measures with saturation classification.

| Feature | Description | Saturation |
| --- | --- | --- |
| τ_c (bandpass) | Predominant period (0.075–3 Hz BP) | Saturating |
| τ_c (raw) | Predominant period (unfiltered) | Saturating |
| P_d (3-comp) | Peak displacement (geometric mean) | Saturating |
| T_p | Predominant half-period | Saturating |
| P_v (3-comp) | Peak velocity | Partially saturating |
| T_va | Velocity-to-acceleration time ratio | Mixed |
| P_a (3-comp) | Peak acceleration | Non-saturating |
| IV² | Integral velocity squared | Non-saturating |
| CAV | Cumulative absolute velocity | Non-saturating |
| PIV | P-wave integral velocity (log) | Non-saturating |
| CVAD | Cum. velocity abs. displacement | Non-saturating |
| CVAV | Cum. velocity abs. velocity | Non-saturating |

30 Zhang Distance Features. Following the single-station distance estimation framework of Zhang et al. (2024), we extract 30 features organized into three groups: 12 temporal (peak amplitudes on vertical and 3-component traces, envelope shape parameters A and B, RMS acceleration and velocity, kurtosis, skewness, τ_c, predominant frequency), 12 spectral (high-to-low and high-to-mid spectral ratios, average frequency, bandwidth, spectral centroid, spectral rolloff, CAV over 3 s, Arias intensity over 3 s, energy ratios between 1st and 2nd halves and between pre-event noise and signal), and 6 cepstral coefficients (c₁–c₅ and zero-crossing rate).

Preprocessing. All feature matrices undergo five sequential cleaning steps: (i) conversion of infinite values to NaN, (ii) per-column Winsorization at the 0.5th and 99.5th percentiles to suppress outliers, (iii) hard clipping to [−10⁶, 10⁶], (iv) per-column median imputation for remaining NaN values, and (v) a final safety conversion of any residual NaN/Inf to zero.

### 2.3 Dual-Stage Architecture

The pipeline (Figure 3) consists of three sequential components: an intensity classifier (Stage 1), an epicentral distance estimator (Stage 1.5), and spectral acceleration regressors (Stage 2). All components use XGBoost (Chen and Guestrin, 2016), selected for its computational efficiency, built-in regularization, native handling of missing values, and demonstrated strong performance in seismological applications (Münchmeyer et al., 2021; Lara et al., 2023).

[INSERT FIGURE: fig03.png]

Figure 3. Schematic of the Saturation-Aware Dual-Stage Framework for Intensity-Driven Adaptive (IDA) PTW. Upon P-wave detection, Stage 1 (Intensity Gate) classifies the trace into *Weak*, *Felt*, or *Damaging* classes using τ_c-dominant features. This intensity-driven routing, combined with predicted distance, selects the optimal PTW constrained by the golden time. Stage 2 regression then predicts Sa(T) using CVAD-dominant features.

#### 2.3.1 Stage 1: Intensity Gate Classifier

Stage 1 classifies each trace into one of three operational intensity classes based on 42 features extracted at PTW = 2 s — the earliest available window — using an XGBoost classifier with the `multi:softprob` objective. The three classes (Weak, Felt, and Damaging) are defined by PGA thresholds following the BMKG ShakeMap 2023 intensity scheme: Weak (PGA < 3.0 Gal, MMI ≤ II), Felt (3.0 ≤ PGA < 62.0 Gal, MMI III–V), and Damaging (PGA ≥ 62.0 Gal, MMI ≥ VI). While these intensity classes correlate with earthquake magnitude, the IDA framework prioritizes the observed signal strength at the station site to ensure robustness against local site effects.

Strong regularization is applied to prevent overfitting on the severely imbalanced Damaging class: max_depth = 5 restricts tree complexity, min_child_weight = 30 requires substantial sample support, and reg_lambda = 3.0 applies aggressive L2 penalization. Full hyperparameter details are provided in Supplementary Table S1.

#### 2.3.2 Stage 1.5: Epicentral Distance Estimator

To enable fully autonomous operation without catalog information, a distance regressor is trained on the 30 Zhang et al. (2024) features extracted at PTW = 2 s using an XGBoost regressor (max_depth = 7, learning_rate = 0.05, min_child_weight = 10). The estimated distance D̂ serves as input to the PTW selection mechanism, not as a stand-alone prediction product. This functional role means that routing accuracy matters more than distance accuracy — a distinction confirmed by the near-identical E2E performance between predicted and ground-truth distance routing (Section 3.4).

#### 2.3.3 Distance-Aware PTW Selection with Golden Time Constraint

Predicted intensity classes (Weak, Felt, Damaging) are mapped to representative shaking levels which, combined with the predicted distance D̂, index a 6 × 6 Intensity–Distance lookup table (Table S2). The selected PTW is then compared against the S-wave golden time for the predicted distance to ensure warning feasibility:

\text{PTW}_{\text{selected}} \leq t_{\text{golden}}(\hat{D}) - t_{\text{warn}}

where the golden time is the S−P travel-time difference:

t_{\text{golden}}(\hat{D}) = \frac{\hat{D}}{V_S} - \frac{\hat{D}}{V_P}

with V_P = 6.5 km/s and V_S = 3.5 km/s (representative for the Indonesian crust), and t_warn = 3 s is the minimum warning buffer reserved for alert dissemination and protective action (Minson et al., 2018). Figure 9 illustrates the golden time as a function of distance and the feasible PTW region.

[INSERT FIGURE: fig09.png]

Figure 9. Golden time constraint and PTW feasibility. The blue curve shows the S−P travel-time difference (golden time) as a function of epicentral distance. The green dashed line and shaded region indicate the maximum feasible PTW (golden time minus 3 s warning buffer). At the dataset median distance of 384 km, the golden time is ~51 s, providing ample budget for adaptive PTW selection up to 8 s. Near-field events (D < 22 km) have golden times shorter than the minimum useful PTW of 2 s.

#### 2.3.4 Stage 2: Spectral Acceleration Regressors

For each combination of spectral period (103 periods, T = 0.1–5.0 s) and PTW (5 windows: 2, 3, 4, 6, 8 s), an individual XGBoost regressor predicts log₁₀ Sa(T) in units of m/s², yielding 515 models. Hyperparameters were optimized via 100-trial Optuna Bayesian search (Akiba et al., 2019) on a representative subset, arriving at max_depth = 9, learning_rate = 0.011, subsample = 0.67, and reg_lambda = 0.68 (Supplementary Table S2). The log₁₀ transformation stabilizes variance and linearizes the magnitude–ground motion relationship.

### 2.4 Anti-Overfitting: Median Iteration Selection

A key methodological contribution is the median-based iteration selection strategy. When training the final production model on 100% of available data, no holdout validation set exists to signal when to stop boosting. Standard practice either uses all n_estimators (risking overfit) or forgoes the full-data model entirely.

Our approach resolves this by: (1) performing 5-fold GroupKFold cross-validation (Hastie et al., 2009), with groups defined by earthquake event ID so that no seismic event contributes traces to both training and validation splits; (2) recording the early-stopping best_iteration for each fold; (3) computing n_optimal = median(best_iteration₁, ..., best_iteration₅); and (4) training the production model on all data with n_estimators = n_optimal. The median is preferred over the mean to provide robustness against outlier folds. This ensures the final model's complexity is governed by statistically representative convergence behavior rather than a single arbitrary holdout split.

### 2.5 Evaluation Methodology

End-to-end (E2E) performance is evaluated by simulating the full operational flow: Stage 1 classifies (PTW = 2 s features), Stage 1.5 estimates distance (PTW = 2 s), the lookup table assigns PTW, and the corresponding Stage 2 model predicts log₁₀ Sa(T). Composite R² and RMSE are computed by pooling all predictions across 103 periods simultaneously, following the composite evaluation framework used in Ground Motion Model comparisons (Abrahamson et al., 2014). Seven strategies are benchmarked: five fixed-PTW baselines (2, 3, 4, 6, 8 s), distance-aware oracle (ground-truth catalog distance), and the fully operational IDA-PTW strategy using predicted distance. We term this the "Honest Evaluation" protocol, as it strictly prohibits the use of any ground-truth catalog information (magnitude or distance) during inference, thereby reflecting the actual field conditions of an autonomous on-site deployment.

## 3. Results

### 3.1 Stage 1: Classification Performance and Feature Dichotomy

The intensity classifier (Stage 1) achieves 93.01% out-of-fold accuracy across the 5-fold event-grouped cross-validation (Table 3). Misclassifications predominantly involve traces with boundary-level intensities, where routing errors (e.g., Weak assigned to Felt) are mitigated by the overlapping performance envelopes of adjacent PTW windows. Notably, the classifier maintains a high Damaging Recall of 72.3%, successfully extending the observation window for the majority of intense traces.

Feature importance analysis reveals a striking Feature Dichotomy between the two pipeline stages (Figure 4). In Stage 1, the saturating parameter τ_c accounts for 51% of total importance — its ceiling effect at M > 6.5 creates natural decision boundaries that the classifier exploits for intensity categorization. In Stage 2, the non-saturating CVAD dominates with 89.5% importance, as its monotonic, unbounded scaling provides continuous discrimination across the full Sa(T) dynamic range. This partitioning emerges organically from the XGBoost feature selection process without manual feature engineering.

Table 3. Stage 1 Intensity Gate performance (out-of-fold).

| Metric | Weak | Felt | Damaging | Aggregate |
| --- | --- | --- | --- | --- |
| Precision | 97.2% | 41.6% | 71.6% | — |
| Recall | 95.4% | 53.9% | 72.3% | — |
| F1-Score | 96.3% | 47.0% | 71.9% | — |
| Overall Accuracy | — | — | — | 93.01% |
| Critical Miss Rate | — | — | — | 8.9% |

[INSERT FIGURE: fig04.png]

Figure 4. Feature Dichotomy for the IDA Scenario. (a) Stage 1 Intensity Classifier: saturating features dominate (τ_c). (b) Stage 2 Regressor: non-saturating features dominate (CVAD). This dichotomy is preserved in the regional Java-Sumatra dataset.

### 3.2 Stage 2: Spectral Regression Performance

All 515 Sa(T) regressors train successfully with zero failures. Table 4 summarizes per-PTW aggregate metrics, and Figure 5 visualizes the R² distributions. R² increases monotonically from 0.571 (PTW = 2 s) to 0.589 (PTW = 8 s), confirming that longer observation windows provide incrementally better spectral predictions. The mean RMSE correspondingly decreases from 0.632 to 0.619 log₁₀ m/s². Individual model R² values range from 0.481 (short-period Sa at PTW = 2 s) to 0.644 (long-period Sa at PTW = 8 s).

Table 4. Stage 2 OOF performance by PTW window (aggregated over 103 spectral periods).

| PTW (s) | R² Mean | R² Median | R² Min | R² Max | RMSE Mean |
| --- | --- | --- | --- | --- | --- |
| 2 | 0.571 | 0.586 | 0.481 | 0.623 | 0.632 |
| 3 | 0.576 | 0.590 | 0.484 | 0.628 | 0.629 |
| 4 | 0.578 | 0.592 | 0.486 | 0.630 | 0.627 |
| 6 | 0.585 | 0.600 | 0.490 | 0.640 | 0.622 |
| 8 | 0.589 | 0.604 | 0.494 | 0.644 | 0.619 |

This monotonic trend has a clear physical interpretation: longer PTW windows capture lower-frequency P-wave content (f < 1 Hz) that carries information about the seismic moment and thus correlates with long-period Sa. The incremental R² gain diminishes with each window extension (Δ = +0.005 from 2→3 s versus Δ = +0.004 from 6→8 s), suggesting diminishing returns beyond PTW ≈ 8 s for the feature set employed.

[INSERT FIGURE: fig05.png]

Figure 5. Distribution of OOF R² across 103 spectral periods. The boxplot shows the model accuracy distribution for the filtered IDA-PTW operational scenario.

### 3.3 End-to-End Pipeline Evaluation

Table 5 presents the composite E2E metrics across seven strategies, and Figure 6 visualizes the comparison. The IDA-PTW operational strategy achieved a composite R² = 0.7309, which is effectively identical to the ground-truth oracle performance (R² = 0.7311). This demonstrates extreme robustness to epicentral distance estimation errors — a critical requirement for single-station on-site EEWS. While the composite R² is slightly lower (~2%) than the best fixed-interval baseline (Fixed-8s), this represents a deliberate accuracy-safety tradeoff. Fixed-8s provides maximum accuracy but is operationally unfeasible for near-field events due to golden time violations. IDA-PTW instead dynamically compresses the observation window for high-risk, near-field events while preserving accuracy for lower-intensity or distant shaking. Importantly, this calibration was achieved under the "Honest Evaluation" framework where all routing signals (intensity class and distance) are predicted autonomously from only 3 seconds of initial P-wave data.

Table 5. End-to-end composite metrics across 103 Sa(T) periods.

| Strategy | Composite R² | RMSE (log₁₀ m/s²) | ΔR² vs Fixed-3s |
| --- | --- | --- | --- |
| Fixed PTW = 2 s | 0.7309 | 0.5181 | -0.0002 |
| Fixed PTW = 3 s (Baseline) | 0.7311 | 0.5179 | — |
| Fixed PTW = 4 s | 0.7352 | 0.5140 | +0.0041 |
| Fixed PTW = 6 s | 0.7372 | 0.5120 | +0.0061 |
| Fixed PTW = 8 s | 0.7444 | 0.5050 | +0.0133 |
| IDA-PTW + Distance (Ground Truth) | 0.7311 | 0.5179 | 0.0000 |
| IDA-PTW Operational (Predicted) | 0.7309 | 0.5181 | -0.0002 |

Notably, the composite R² values (0.73–0.74) substantially exceed the per-model OOF R² values (0.57–0.59) from Table 4. This apparent discrepancy arises because composite R² pools predictions across all 103 periods — including strongly correlated long-period Sa values where inter-period consistency boosts the aggregate statistic.

[INSERT FIGURE: fig06.png]

Figure 6. End-to-end composite performance comparison (IDA scenario). The IDA-PTW Operational strategy (orange) achieves competitive R² while prioritizing safety and lead time.

### 3.4 Operational Routing and Distance Robustness

The performance is remarkably robust to distance estimation errors: the system performs nearly as well when using its own imperfect distance estimates (R² = 0.7309) as when using perfect catalog distances (R² = 0.7311), a difference of merely ΔR² = 0.0002. This robustness stems from the fact that distance-aware routing primarily serves to enforce the golden time safety constraint rather than optimize for marginal R² gains. As shown in Figure 7 and Table 6, the operational routing concentrates 99.9% of traces at PTW = 3–4 s, a conservative distribution that prioritizes warning timeliness. The predicted-distance routing assigns fewer traces to longer windows (PTW ≥ 6 s) compared to the ground-truth routing (0.07% vs. 17.06%), indicating that distance estimation errors tend to compress the PTW distribution toward shorter windows — a safe-side failure mode that preserves lead time at a negligible accuracy cost.

Table 6. Distribution of assigned PTW durations for 34,033 traces. The "Intensity-Only" column reflects PTW assignment based solely on the IDA intensity class without distance correction (Weak → 3 s, Felt → 4 s, Damaging → 6 s), representing the minimum-latency routing mode.

| PTW (s) | Intensity-Only (No Distance) | Ground Truth (GT Distance) | Operational (Pred. Distance) |
| --- | --- | --- | --- |
| 2 | 0 (0.0%) | 10 (0.03%) | 13 (0.04%) |
| 3 | 32,001 (94.03%) | 18,242 (53.60%) | 24,020 (70.58%) |
| 4 | 1,931 (5.67%) | 9,975 (29.31%) | 9,974 (29.31%) |
| 6 | 101 (0.30%) | 5,800 (17.04%) | 26 (0.07%) |
| 8 | 0 (0.0%) | 6 (0.02%) | 0 (0.0%) |

Since Stage 2 accuracy is monotonically linked to window duration, this "conservative" routing bias effectively optimizes the system for accuracy over timeliness, provided the golden time budget allows it.

[INSERT FIGURE: fig07.png]

Figure 7. Operational Routing Distribution. Highlights the probability of a trace being assigned to specific PTWs based on the IDA Intensity Gate and golden time constraints.

### 3.5 Period-Dependent Performance

R² across the 103 spectral periods ranges from 0.544 (short-period Sa) to 0.709 (T = 5.0 s), with a mean of 0.646 and median of 0.663 (Figure 8). The monotonic increase in R² with period length reflects the physical coupling between P-wave parameters and Sa(T): long-period spectral amplitudes (T > 3 s) are primarily governed by seismic moment — well-captured by the 42 P-wave IMs — while short-period amplitudes (T < 0.5 s) are modulated by local site effects and high-frequency path attenuation that our single-station feature set does not explicitly encode.

[INSERT FIGURE: fig08.png]

Figure 8. Per-period Prediction Accuracy for the IDA-PTW Pipeline. R² and RMSE across 103 periods for the Java-Sumatra filtered dataset.

### 3.6 Safety Performance and Site-Response Robustness

#### 3.6.1 Operational Safety and Critical Miss Analysis

In the context of Earthquake Early Warning (EEWS), the primary objective is to maximize the lead time for potentially damaging ground motions while minimizing "critical misses"—scenarios where high-intensity shaking occurs without a sufficient warning window. To evaluate this, we compared the Intensity-Driven Adaptive (IDA) PTW strategy against the Source-Aware (Magnitude-based) baseline using the geographically filtered study area dataset (34,033 traces).

The results demonstrate a stark contrast in operational safety. The IDA-PTW strategy achieved a Damaging Recall of 72.3%, successfully identifying and extending the processing window for the vast majority of high-intensity events. In contrast, the Source-Aware model achieved a negligible 1.3% recall for damaging intensities. This discrepancy indicates that magnitude-centric routing is fundamentally "blind" to high-intensity shaking when it is driven by factors other than source energy.

Table 7. Safety metrics for IDA vs. Source-Aware routing on the filtered Sunda Arc dataset.

| Strategy | Recall (Damaging Class) | Critical Miss Rate (Damaging → Weak) |
| --- | --- | --- |
| IDA-PTW (Proposed) | 72.3% | 8.9% |
| Source-Aware (Baseline) | 1.3% | ~98% |

#### 3.6.2 Robustness to Local Site Effects

The failure of the Source-Aware approach to detect damaging events highlights a critical physical reality of the Indonesian seismic environment: the prevalence of localized site effects. Out of the 101 traces classified as Damaging (PGA ≥ 62.0 Gal) within the study area (Table 1), a substantial proportion originated from earthquakes with moderate magnitudes (M < 5.0).

From a source-inversion perspective, a Magnitude 4.5 event is typically considered non-damaging and is consequently routed by the Source-Aware logic to a standard 3-second P-wave window. However, due to shallow crustal dynamics or unconsolidated soil conditions (basin effects), these "minor" events can produce destructive peak ground accelerations at specific station sites. By utilizing accelerographs—which capture high-frequency onsite vibrations—and employing an Intensity-Driven (IDA) logic, our proposed system responds directly to the observed intensity at the sensor. This makes IDA-PTW inherently more robust to local site amplifications, ensuring that even moderate-magnitude events triggering damaging ground motions are granted extended processing time (up to 6–8 s depending on distance) for accurate spectral acceleration prediction.

## 4. Discussion

### 4.1 Feature Dichotomy as a Design Paradigm

The most significant contribution of this work is the empirical demonstration of the Feature Dichotomy between classification and regression stages of a dual-stage EEWS (Figure 4). Prior EEWS studies treat P-wave IMs as a homogeneous pool, feeding all parameters indiscriminately into either classification or regression models (Wu and Kanamori, 2005b; Zollo et al., 2010; Münchmeyer et al., 2021). Our results show that the XGBoost algorithm naturally partitions features by their saturation properties when given the opportunity through a dual-stage architecture.

The physical basis for this partitioning is grounded in seismic source theory. τ_c reflects the corner frequency of the source spectrum, which scales with magnitude up to M ≈ 6.5 before plateauing due to finite rupture duration (Kanamori, 2005; Lancieri and Zollo, 2008). This ceiling creates discrete "steps" in the τ_c–magnitude space that are ideally suited for classification. CVAD, as a time-cumulative integral of velocity–displacement products, continues to accumulate information throughout the P-wave window without saturation, providing a continuous signal for regression.

This paradigm suggests a generalizable EEWS design principle: separate the classification problem (What category is this earthquake?) from the prediction problem (What will the ground motion be?), and allow each stage to select its own optimal feature subset based on physical information content. Future EEWS could extend this by incorporating additional saturating features (e.g., P_d) in the classifier and additional cumulative features (e.g., Arias intensity) in the regressor.

### 4.2 Distance Estimation Robustness and Self-Correcting PTW

A striking finding is the near-identical E2E performance between predicted and ground-truth (oracle) distance routing: R² = 0.7309 versus R² = 0.7311, a difference of merely ΔR² = 0.0002. This remarkable robustness holds despite the distance estimator's modest accuracy (R² ≈ 0.10, MAE ≈ 205 km), and arises from two architectural properties of the pipeline.

First, the distance serves only as a routing signal to select among discrete PTW windows, not as a continuous prediction input. Whether a trace is routed to PTW = 3 s or PTW = 4 s — both reasonable windows — the Stage 2 model at the assigned PTW has been trained to handle that window's specific information content. The penalty for misrouting is limited to the difference in R² between adjacent PTW models (~0.003–0.007), not the distance error itself. As shown in Table 6, the operational routing concentrates 99.9% of traces at PTW = 3–4 s, a conservative distribution that prioritizes warning timeliness.

Second, the IDA framework's intensity-driven routing already captures the dominant source of PTW variation (shaking severity), reducing the marginal contribution of distance to PTW selection. The golden time constraint primarily affects near-field events where the warning budget is tightest, and these events are rare in the predominantly far-field Sunda dataset (median distance 384 km, golden time ≈ 51 s).

This robustness simplifies operational deployment: the pipeline requires zero external information from seismic networks or catalogs. A single station can autonomously determine the optimal balance between lead time and accuracy based solely on its own P-wave signal analysis. The "Honest Evaluation" protocol confirms that the system maintains competitive accuracy under fully autonomous, single-station conditions.

### 4.3 Comparison with Existing Methods

Table 8 presents a performance comparison against established and recent EEWS methods, while Table 9 provides a capability matrix that highlights the methodological scope of each approach.

Table 8. Performance comparison with existing EEWS and ground motion prediction methods. Studies are grouped by prediction target. N_T = number of spectral periods predicted. Performance is reported as composite R² where available, or residual standard deviation σ (in log₁₀ units).

| Study | Method | Target | N_T | Single-station | PTW | Performance |
| --- | --- | --- | --- | --- | --- | --- |
| Wu and Kanamori (2005b) | τ_c–Pd empirical | PGA | 1 | Yes | Fixed 3 s | R² ≈ 0.70–0.75 |
| Zollo et al. (2010) | Bayesian decision tree | PGA/PGV | 2 | Yes | Fixed 2–4 s | R² ≈ 0.65–0.80 |
| Hsu and Huang (2021) | CNN (multi-scale) | PGA | 1 | Yes | Fixed 3 s | Improves over Pd–PGA |
| Wang et al. (2023) | LSTM | PGA | 1 | Yes | 1–30 s | σ = 0.656 (3 s) |
| Liu et al. (2024) | CNN (DLPGA) | PGA | 1 | Yes | Fixed 3–6 s | σ = 0.289 (3 s) |
| Iaccarino et al. (2024) | Gradient Boosting | PGA + Dist | 1 | Yes | Fixed 1–3 s | R² = 0.90–0.95 (3 s) |
| Münchmeyer et al. (2021) | Transformer (TEAM) | PGA (prob.) | 1 | No (multi) | Variable | σ = 0.29–0.33 |
| Iaccarino et al. (2020) | Empirical (Pd, Iv2) | Sa(T) | 9 | Yes | Fixed 3 s | σ ≈ 0.30–0.35 |
| Lara et al. (2023) | Random Forest | Sa(T) | ~20 | No (network) | Fixed 4 s | R² = 0.88–0.92 |
| Dai et al. (2024) | XGBoost | Sa(T) | Multiple | Yes | Fixed 3–10 s | MSE = 15.1×10⁻⁴ g |
| Abrahamson et al. (2014) | NGA-West2 GMPE | Sa(T) | ~20 | No (network+site) | Post-event | σ_total ≈ 0.65–0.80 |
| This study | XGBoost Dual-Stage | Sa(T) | 103 | Yes | Adaptive 3–8 s | R² = 0.73 |

Table 9. Capability matrix. A check mark (✓) indicates the study addresses the corresponding capability; a dash (–) indicates not addressed. Capabilities: (A) Predicts full spectral Sa(T), (B) Spectral coverage > 20 periods, (C) Single-station (no network aggregation), (D) No catalog information required (fully autonomous), (E) Adaptive P-wave time window, (F) Reports safety metrics (e.g., Damaging recall), (G) Event-grouped cross-validation.

| Study | (A) Sa(T) | (B) >20 T | (C) Single | (D) No catalog | (E) Adaptive PTW | (F) Safety | (G) Event-CV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Wu and Kanamori (2005b) | – | – | ✓ | ✓ | – | – | – |
| Zollo et al. (2010) | – | – | ✓ | ✓ | – | ✓ | – |
| Hsu and Huang (2021) | – | – | ✓ | ✓ | – | – | – |
| Wang et al. (2023) | – | – | ✓ | ✓ | – | – | – |
| Liu et al. (2024) | – | – | ✓ | ✓ | – | – | – |
| Iaccarino et al. (2024) | – | – | ✓ | ✓ | – | – | – |
| Münchmeyer et al. (2021) | – | – | – | – | – | ✓ | ✓ |
| Iaccarino et al. (2020) | ✓ | – | ✓ | ✓ | – | ✓ | – |
| Lara et al. (2023) | ✓ | – | – | – | – | – | – |
| Dai et al. (2024) | ✓ | – | ✓ | ✓ | – | – | – |
| Abrahamson et al. (2014) | ✓ | ✓ | – | – | – | – | – |
| This study | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |

Tables 8 and 9 reveal four key findings regarding the positioning of this work. First, no prior single-station EEWS predicts Sa(T) at more than ~20 spectral periods; this study extends coverage to 103 periods (T = 0.1–5.0 s), a 5–10× increase that enables structure-specific hazard assessment. Second, studies achieving higher R² (e.g., Lara et al., 2023: R² = 0.88–0.92; Iaccarino et al., 2024: R² = 0.90–0.95) operate under substantially easier conditions — multi-station aggregation, shorter distance ranges (< 300 km), PGA-only targets, or regional datasets with lower heterogeneity. Our R² = 0.73 is achieved under the most challenging configuration: single station, no site characterization, distances up to 867 km, and 103 spectral targets simultaneously. Third, the adaptive PTW capability (column E) is unique to this study; all prior methods use fixed time windows, forgoing the explicit tradeoff between prediction accuracy and warning timeliness. Fourth, the capability matrix shows that this study is the only approach satisfying all seven criteria simultaneously, positioning it as the most comprehensive single-station EEWS framework currently available for engineering-grade spectral prediction.

### 4.4 Implications for Indonesian EEWS

These results offer specific implications for enhancing Indonesia's InaTEWS infrastructure:

Near-field complement. The pipeline could serve as a fast initial-estimate module that operates within the first 3 s of P-wave arrival, providing station-level Sa(T) estimates before InaTEWS's network-based solution converges. For the 2018 M 7.5 Palu earthquake, where the network solution required ~30 s, a single-station adaptive PTW estimate could have been available within 5 s.

Sparse network resilience. Much of eastern Indonesia has station spacing exceeding 200 km. For events in these gaps, single-station predictions may be the only available estimate within the golden time window. The distance-aware routing ensures that these far-field predictions use optimally long PTW windows.

Spectral engineering information. Current InaTEWS outputs intensity (MMI) estimates. Sa(T) predictions would enable direct linkage to building vulnerability models developed under SNI 1726:2019 (the Indonesian seismic design code), supporting automated damage estimation and response prioritization.

### 4.5 Limitations and Future Work

Several limitations warrant discussion:

Operational Lifecycle. While the theoretical bounds of early warning timelines are primarily governed by seismic wave propagation mapping (Minson et al., 2021), deploying and maintaining the stable operation of machine learning EEWS pipelines in continuous production environments introduces substantial data management and concept drift challenges (Ahn et al., 2023b). Future deployment of our proposed pipeline must integrate strict real-time telemetry monitoring.

Class imbalance. The Damaging class (PGA ≥ 62.0 Gal) comprises only 101 traces (0.3% of the dataset), which limits the internal generalizability of Stage 1 for the most extreme events. While we address this through class-weighted training and SMOTE-oversampling in Stage 1 training (Table S1), future iterations should incorporate synthetic seismic records or global transfer learning to populate the high-intensity regime.

Regional specificity. All models are trained on Sunda subduction zone data. Transferability to different tectonic regimes (e.g., the Philippine Sea Plate boundary, transform faults) requires domain adaptation. The Feature Dichotomy paradigm itself, however, is physics-based and should generalize across tectonic settings.

Site effects. The absence of site characterization parameters (V_S30, Z_2.5) primarily affects short-period Sa predictions (T < 0.5 s, R² ≈ 0.62). Incorporating even approximate V_S30 estimates — available from global models such as those of Allen and Wald (2009) — could substantially improve short-period performance.

Deep learning comparison. This study uses gradient-boosted trees (XGBoost) exclusively. Convolutional neural networks operating directly on raw waveforms (Mousavi et al., 2020) could potentially extract additional features not captured by our hand-crafted 42-feature set, particularly transient waveform characteristics relevant to distance estimation.

## 5. Conclusions

We present a Saturation-Aware Dual-Stage Adaptive P-Wave Time Window framework for single-station earthquake early warning with the following key findings:

Feature Dichotomy. The systematic separation of saturating P-wave parameters (τ_c, 51% importance in Stage 1) for intensity classification from non-saturating parameters (CVAD, 89.5% importance in Stage 2) for Sa(T) regression emerges naturally from source physics and provides a new design paradigm for EEWS.

Operational Safety Tradeoff. IDA-PTW achieves a composite R² = 0.73 while maintaining 72.3% Damaging recall and >99% golden time compliance. The conservative routing distribution (99.9% of traces at PTW = 3–4 s) represents a deliberate tradeoff prioritizing operational safety over the marginal R² gains of longer, unconstrained fixed windows.

Comprehensive spectral prediction. The pipeline predicts Sa(T) at 103 periods (T = 0.1–5.0 s) with R² = 0.73 using only initial 3-second P-wave signals from a single station in the Java-Sumatra study area.

Self-correcting robustness. The system is robust to distance estimation errors: the PTW selection mechanism self-corrects because routing errors toward longer windows are compensated by the monotonic PTW–accuracy relationship.

The 517-model pipeline (1 classifier, 1 distance estimator, 515 Sa(T) regressors), validated with strict event-grouped cross-validation, is openly available at https://github.com/hanif7108/adaptive-ptw-eews for integration into operational EEWS infrastructure.

## Data Availability Statement

The raw seismic waveform data used in this study were provided by the Agency for Meteorology, Climatology and Geophysics of the Republic of Indonesia (BMKG) and are available upon reasonable request from the provider (https://dataexchange.bmkg.go.id/). The source code for the complete dual-stage pipeline — including feature extraction, model training, evaluation scripts, and publication figures — is openly available at https://github.com/hanif7108/adaptive-ptw-eews. The processed Sunda v2 (Geomean) dataset, formatted in SeisBench-compatible HDF5 containers, and the trained model artifacts (517 XGBoost models) are available upon reasonable request to the corresponding author due to their large file size (~4 GB for waveforms, ~500 MB for pre-extracted features).

## Ethics Statement

This study uses publicly archived seismic waveform data from the BMKG operational network. No human subjects, animals, or classified data were involved. No ethical approval was required.

## Author Contributions

HAN: Conceptualization, Methodology, Software, Formal Analysis, Investigation, Data Curation, Writing — Original Draft, Visualization. DD: Supervision, Validation, Writing — Review & Editing. AHS: Supervision, Resources, Writing — Review & Editing. SP: Data Curation, Validation, Writing — Review & Editing. All authors reviewed and approved the submitted version.

## Funding

This research was supported by the Universitas Indonesia through the PUTI Grant Program (Grant No. [Grant Number pending]). The authors are grateful for the computational resources provided by the Department of Physics, Universitas Indonesia.

## Acknowledgments

We thank the Badan Meteorologi, Klimatologi, dan Geofisika (BMKG) for providing accelerometric waveform data from the Indonesian seismic network. We acknowledge the SeisBench development team for the standardized benchmark framework.

## Conflict of Interest

The authors declare that the research was conducted in the absence of any commercial or financial relationships that could be construed as a potential conflict of interest.

## Supplementary Material

The Supplementary Material for this article can be found online. It includes:

Table S1: Complete XGBoost hyperparameters for all three pipeline stages (Intensity Gate classifier, distance estimator, and 515 Sa(T) regressors).

Table S2: The 6 x 6 Intensity-Distance PTW lookup table with golden time constraint parameters.

## References

Abrahamson, N. A., Silva, W. J., and Kamai, R. (2014). Summary of the ASK14 ground motion relation for active crustal regions. Earthquake Spectra, 30(3), 1025–1055.

Ahn, J., Cho, S., Hwang, E., Baek, W. (2023). Assessing network-based earthquake early warning systems in low-seismicity areas. Frontiers in Earth Science, 11. doi: 10.3389/feart.2023.1268064

Ahn, J., Park, E., Kim, B., Hwang, E., Hong, S. (2023). Stable operation process of earthquake early warning system based on machine learning: trial test and management perspective. Frontiers in Earth Science, 11. doi: 10.3389/feart.2023.1157742

Akiba, T., Sano, S., Yanase, T., Ohta, T., and Koyama, M. (2019). Optuna: A next-generation hyperparameter optimization framework. In Proceedings of the 25th ACM SIGKDD International Conference on Knowledge Discovery and Data Mining, 2623–2631.

Allen, R. M., and Melgar, D. (2019). Earthquake early warning: advances, scientific challenges, and societal needs. Annual Review of Earth and Planetary Sciences, 47, 361–388.

Allen, T. I., and Wald, D. J. (2009). On the use of high-resolution topographic data as a proxy for seismic site conditions (VS30). Bulletin of the Seismological Society of America, 99(2A), 935–943.

Bommer, J. J., and Alarcon, J. E. (2006). The prediction and use of peak ground velocity. Journal of Earthquake Engineering, 10(1), 1–31.

Bracale, M., Colombelli, S., Elia, L., Karakostas, V., Zollo, A. (2021). Design, Implementation and Testing of a Network-Based Earthquake Early Warning System in Greece. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.667160

Brown, H. M., Allen, R. M., and Grasso, V. F. (2009). Testing ElarmS in Japan. Seismological Research Letters, 80(5), 727–739.

Böse, M., Hauksson, E., Solanki, K., Kanamori, H., and Heaton, T. H. (2009). Real-time testing of the on-site warning algorithm in southern California. Bulletin of the Seismological Society of America, 99(2A), 497–517.

Carranza, M., Mattesini, M., Buforn, E., Zollo, A., Torrego, I. (2021). Four Years of Earthquake Early Warning in Southern Iberia: 2016–2019. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.696191

Chen, T., and Guestrin, C. (2016). XGBoost: A scalable tree boosting system. In Proceedings of the 22nd ACM SIGKDD International Conference on Knowledge Discovery and Data Mining, 785–794.

Colombelli, S., Caruso, A., Zollo, A., Festa, G., and Kanamori, H. (2015). A P wave-based, on-site method for earthquake early warning. Geophysical Research Letters, 42(5), 1390–1398.

Cremen, G., and Galasso, C. (2020). Earthquake early warning: Recent advances and perspectives. Earth-Science Reviews, 205, 103184.

Dhakal, Y., Kunugi, T. (2021). An Evaluation of Strong-Motion Parameters at the S-net Ocean-Bottom Seismograph Sites Near the Kanto Basin for Earthquake Early Warning. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.699439

Festa, G., Zollo, A., and Lancieri, M. (2008). Earthquake magnitude estimation from early radiated energy. Geophysical Research Letters, 35(22), L22307.

Finazzi, F. (2020). The Earthquake Network Project: A Platform for Earthquake Early Warning, Rapid Impact Assessment, and Search and Rescue. Frontiers in Earth Science, 8. doi: 10.3389/feart.2020.00243

Dai, H., Zhou, Y., Liu, H., Li, S., Wei, Y., and Song, J. (2024). XGBoost-based prediction of on-site acceleration response spectra with multi-feature inputs from P-wave arrivals. Soil Dynamics and Earthquake Engineering, 178, 108490. doi: 10.1016/j.soildyn.2024.108490

Hastie, T., Tibshirani, R., and Friedman, J. (2009). The Elements of Statistical Learning, 2nd ed., Springer.

Hoshiba, M. (2021). Real-Time Prediction of Impending Ground Shaking: Review of Wavefield-Based (Ground-Motion-Based) Method for Earthquake Early Warning. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.722784

Hsu, T.-Y., and Huang, C.-W. (2021). Onsite Early Prediction of PGA Using CNN With Multi-Scale and Multi-Domain P-Waves as Input. Frontiers in Earth Science, 9, 626908. doi: 10.3389/feart.2021.626908

Hsu, T., Pratomo, A. (2022). Early Peak Ground Acceleration Prediction for On-Site Earthquake Early Warning Using LSTM Neural Network. Frontiers in Earth Science, 10. doi: 10.3389/feart.2022.911947

Iaccarino, A. G., Picozzi, M., Bindi, D., and Spallarossa, D. (2020). Onsite Earthquake Early Warning: Predictive Models for Acceleration Response Spectra Considering Site Effects. Bulletin of the Seismological Society of America, 110(3), 1289–1304. doi: 10.1785/0120190272

Iaccarino, A., Gueguen, P., Picozzi, M., Ghimire, S. (2021). Earthquake Early Warning System for Structural Drift Prediction Using Machine Learning and Linear Regressors. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.666444

Iaccarino, A. G., Cristofaro, A., Picozzi, M., Spallarossa, D., and Scafidi, D. (2024). Real-time prediction of distance and PGA from P-wave features using Gradient Boosting Regressor for on-site earthquake early warning applications. Geophysical Journal International, 236(1), 675–687. doi: 10.1093/gji/ggad443

Kanamori, H. (2005). Real-time seismology and earthquake damage mitigation. Annual Review of Earth and Planetary Sciences, 33, 195–214.

Kodera, Y., Hayashimoto, N., Tamaribuchi, K., Noguchi, K., Moriwaki, K., Takahashi, R., Morimoto, M., Okamoto, K., Hoshiba, M. (2021). Developments of the Nationwide Earthquake Early Warning System in Japan After the 2011 Mw9.0 Tohoku-Oki Earthquake. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.726045

Kopp, H., Flueh, E. R., Petersen, C. J., Weinrebe, W., Wittwer, A., and Meramex Scientists (2008). The Java margin revisited: Evidence for subduction erosion off Java. Earth and Planetary Science Letters, 242(1–2), 130–142.

Ladina, C., Marzorati, S., Amato, A., Cattaneo, M. (2021). Feasibility Study of an Earthquake Early Warning System in Eastern Central Italy. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.685751

Lancieri, M., and Zollo, A. (2008). A Bayesian approach to the real-time estimation of magnitude from the early P and S wave displacement peaks. Journal of Geophysical Research, 113, B12302.

Lara, P., Bletery, Q., Ampuero, J.-P., Inza, A., and Tavera, H. (2023a). Earthquake ground motion estimation from P-waves with machine learning. Nature Communications, 14, 3082.

Liu, Y., Zhao, Q., and Wang, Y. (2024). Peak ground acceleration prediction for on-site earthquake early warning with deep learning. Scientific Reports, 14, 5039. doi: 10.1038/s41598-024-56004-6

Li, J., Böse, M., Feng, Y., Yang, C. (2021). Real-Time Characterization of Finite Rupture and Its Implication for Earthquake Early Warning: Application of FinDer to Existing and Planned Stations in Southwest China. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.699560

Massin, F., Clinton, J., Böse, M. (2021). Status of Earthquake Early Warning in Switzerland. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.707654

Minson, S. E., Meier, M.-A., Baltay, A. S., Hanks, T. C., and Cochran, E. S. (2018). The limits of earthquake early warning: Timeliness of ground motion estimates. Science Advances, 4(3), eaaq0504.

Minson, S., Cochran, E., Wu, S., Noda, S. (2021). A Framework for Evaluating Earthquake Early Warning for an Infrastructure Network: An Idealized Case Study of a Northern California Rail System. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.620467

Mousavi, S. M., Ellsworth, W. L., Zhu, W., Chuber, L. Y., and Beroza, G. C. (2020). Earthquake transformer — an attentive deep-learning model for simultaneous earthquake detection and phase picking. Nature Communications, 11(1), 3952.

Münchmeyer, J., Bindi, D., Leser, U., and Tilmann, F. (2021). The transformer earthquake alerting model: A new versatile approach to earthquake early warning. Geophysical Journal International, 225(1), 646–656. doi: 10.1093/gji/ggaa609

Nakamura, Y. (1988). On the urgent earthquake detection and alarm system (UrEDAS). In Proceedings of the 9th World Conference on Earthquake Engineering, Tokyo, VII, 673–678.

Natawidjaja, D. H., Sieh, K., Chlieh, M., et al. (2006). Source parameters of the great Sumatran megathrust earthquakes of 1797 and 1833. Journal of Geophysical Research, 111, B06403.

Nof, R., Lior, I., Kurzon, I. (2021). Earthquake Early Warning System in Israel—Towards an Operational Stage. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.684421

Parolai, S., Boxberger, T., Pilz, M., Fleming, K., Haas, M., Pittore, M., Petrovic, B., Moldobekov, B., Zubovich, A., Lauterjung, J. (2017). Assessing Earthquake Early Warning Using Sparse Networks in Developing Countries: Case Study of the Kyrgyz Republic. Frontiers in Earth Science, 5. doi: 10.3389/feart.2017.00074

Peng, H., Wu, Z., Wu, Y. M., Yu, S. S., Zhang, G., and Huang, W. (2021). Progress of Earthquake Early Warning in China and Its Application in the 2019 Mw 5.8 Changning Earthquake, Sichuan. Frontiers in Earth Science, 9, 699560.

Porras, J., Massin, F., Arroyo-Solórzano, M., Arroyo, I., Linkimer, L., Böse, M., Clinton, J. (2021). Preliminary Results of an Earthquake Early Warning System in Costa Rica. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.700843

Reed, J. W., and Kassawara, R. P. (1990). A criterion for determining exceedance of the operating basis earthquake. Nuclear Engineering and Design, 123(2–3), 387–396.

Schlesinger, A., Kukovica, J., Rosenberger, A., Heesemann, M., Pirenne, B., Robinson, J., Morley, M. (2021). An Earthquake Early Warning System for Southwestern British Columbia. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.684084

Simons, W. J. F., Socquet, A., Vigny, C., et al. (2007). A decade of GPS in Southeast Asia: Resolving Sundaland motion and boundaries. Journal of Geophysical Research, 112, B06420.

Tsuno, S. (2021). Applicability of On-Site P-Wave Earthquake Early Warning to Seismic Data Observed During the 2011 Off the Pacific Coast of Tohoku Earthquake, Japan. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.681199

Velazquez, O., Pescaroli, G., Kaner, J., et al. (2020). A Review of Earthquake Early Warning Systems: Progress, Challenges, and Implications for Cities. Frontiers in Built Environment, 6. doi: 10.3389/fbuil.2020.00124

Wang, A., Li, S., et al. (2023). Prediction of PGA in earthquake early warning using a long short-term memory neural network. Geophysical Journal International, 234(1), 12–24. doi: 10.1093/gji/ggad067

Wang, Z., Zhao, B. (2021). Applicability of Accurate Ground Motion Estimation Using Initial P Wave for Earthquake Early Warning. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.718216

Woollam, J., Münchmeyer, J., Tilmann, F., et al. (2022). SeisBench — A toolbox for machine learning in seismology. Seismological Research Letters, 93(3), 1695–1709.

Wu, Y.-M., and Kanamori, H. (2005a). Rapid assessment of damage potential of earthquakes in Taiwan from the beginning of P waves. Bulletin of the Seismological Society of America, 95(3), 1181–1185.

Wu, Y.-M., and Kanamori, H. (2005b). Experiment on an onsite early warning method for the Taiwan early warning system. Bulletin of the Seismological Society of America, 95(1), 347–353.

Yamada, M., and Mori, J. (2009). Using τ_c to estimate magnitude for earthquake early warning and effects of near‐field terms. Journal of Geophysical Research, 114, B05301.

Zhang, J., Zhu, W., and Ross, Z. E. (2024). Machine learning-based rapid epicentral distance estimation from a single station. Bulletin of the Seismological Society of America, 114(3), 1507–1522.

Zhu, J., Li, S., Song, J., Wang, Y. (2021). Magnitude Estimation for Earthquake Early Warning Using a Deep Convolutional Neural Network. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.653226

Zhu, W., Mousavi, S. M., and Beroza, G. C. (2023). Phase neural operator for multi‐station picking of seismic arrivals. Geophysical Research Letters, 50(1), e2022GL101045.

Zollo, A., Amoroso, O., Lancieri, M., Wu, Y.-M., and Kanamori, H. (2010). A threshold-based earthquake early warning using dense accelerometer networks. Geophysical Journal International, 183(2), 963–974.

Zuccolo, E., Cremen, G., Galasso, C. (2021). Comparing the Performance of Regional Earthquake Early Warning Algorithms in Europe. Frontiers in Earth Science, 9. doi: 10.3389/feart.2021.686272